AUTOMATED CABLE SPECIMEN PREPARATION SYSTEM

1. Existing Technology and Current State

Manual Specimen Preparation

The conventional cable specimen preparation process is largely manual and involves multiple operations such as cutting, straightening, insulation/sheath stripping, slicing, and specimen formation. A typical specimen may require approximately 10 minutes to prepare, with significant operator-dependent variability and errors.

Industrial Wire Stripping Machines

Commercial wire-processing and stripping machines can perform high-speed cutting and insulation stripping. However, these systems are primarily designed for wire processing and generally do not provide an integrated solution for test-specimen preparation, dimensional verification, or standards-specific specimen formation.

Existing Automated Prototype

A recently reported automated cable specimen preparation prototype integrates multiple preparation stages and reduces the preparation time to approximately 142 seconds per specimen. The reported system demonstrates good repeatability, with a Cpk value of approximately 1.71. However, the system does not incorporate comprehensive real-time material sensing, adaptive cutting-depth control, or intelligent closed-loop quality verification.

2. Major Research and Technology Gaps

| Technology Gap | Current State | Proposed R&D Opportunity |
| --- | --- | --- |
| Cable Diameter Sensing | Manual measurement using a micrometer or no automated measurement | Integrate a laser diameter gauge for continuous, non-contact measurement and automatic blade-position adjustment |
| Material Identification | Material is generally identified manually or based on operator knowledge | Use spectral sensing and machine-learning methods to identify materials such as PVC, XLPE, and HDPE |
| Adaptive Cutting Depth | Fixed blade depth or manual adjustment | Implement closed-loop cutting-depth control using ultrasonic sensing and encoder feedback |
| Integrated Specimen Preparation | Individual machines or manual operations are used for different stages | Develop a single automated system integrating feeding, cutting, stripping, flattening, slicing, and specimen formation |
| Standards Compliance | Parameters and specimen dimensions depend heavily on operator interpretation | Implement automatic selection of specimen dimensions and processing parameters according to applicable IS standards |

3. Proposed Novelty

The proposed system aims to integrate multiple technologies into a single adaptive and standards-aware automated cable specimen preparation platform.

The key novelty is the combination of:

- Real-time cable diameter measurement and material sensing

- Adaptive blade-depth control to minimize conductor damage

- Automated preparation of standards-compliant dumbbell specimens

- Inline dimensional and surface-quality verification before final specimen release

- Industry 4.0 connectivity for data logging, traceability, and laboratory integration

- RFID-based specimen identification and traceability

- Integration with LIMS for automated test-data management

Rather than simply automating individual cutting or stripping operations, the proposed system focuses on closed-loop, sensor-driven specimen preparation with automated quality assurance and standards-based parameter selection.

4. Recommended System Architecture

The proposed system can be organized into five major processing stages.

Stage 1 – Feed and Measure

The cable specimen is automatically fed into the system using controlled rollers.

A laser diameter sensor continuously measures the cable diameter. At the same stage, a material-identification module can analyze the cable insulation/sheath characteristics and classify the material, such as:

- PVC

- XLPE

- HDPE

- Other relevant polymeric insulation/sheath materials

The measured diameter and identified material are used by the controller to determine suitable processing parameters.

Stage 2 – Cutting and Adaptive Stripping

A servo-controlled cutting mechanism performs the required cutting and stripping operations.

The system automatically adjusts the blade position according to the measured cable diameter and material properties. Ultrasonic sensing and encoder feedback can be used to monitor the cutting process and provide closed-loop control of blade depth.

The objective is to remove the insulation or sheath while minimizing the possibility of conductor damage or unintended material removal.

Stage 3 – Stripping and Flattening

After stripping, the specimen is transferred through a controlled roller mechanism.

Reverse-roller or controlled-roller motion can be used to straighten and flatten the prepared material. A controlled weighting or pressing mechanism can provide consistent flattening conditions.

This stage aims to reduce operator-dependent variation before the specimen proceeds to slicing and final specimen formation.

Stage 4 – Slicing and Dumbbell Specimen Formation

The prepared material is positioned accurately for longitudinal slicing.

A precision longitudinal cutting mechanism separates the required specimen section, after which an automated dumbbell-shaped specimen forming/stamping mechanism produces the required specimen geometry.

The die or forming parameters can be selected automatically according to the applicable testing standard and specimen type.

Stage 5 – Inline Verification

Before the specimen is released for testing, an industrial camera-based vision system performs automated quality inspection.

The system can verify parameters such as:

- Specimen length

- Width

- Thickness-related dimensional features

- Gauge length

- Dumbbell geometry

- Cutting-edge quality

- Visible conductor damage

- Surface defects

- Alignment and positioning

Specimens that satisfy the programmed acceptance criteria are released automatically, while defective specimens are identified and rejected or flagged for operator review.

5. Target Performance and Validation Parameters

The proposed system can be evaluated against the following target performance indicators:

| Parameter | Proposed Target | Reference/Current State |
| --- | --- | --- |
| Cycle Time | < 120 seconds/specimen | ~600 seconds manual; ~142 seconds reported prototype |
| Process Capability (Cpk) | > 2.0 | ~1.71 reported prototype |
| Specimen Rejection Rate | < 1% | Higher variability in manual preparation |
| Dimensional Accuracy | To be established through standards-based validation | Operator-dependent in manual preparation |
| Conductor Damage | Zero visible conductor nicks | Major quality concern during stripping |
| Emergency Stop Response | < 50 ms target | To be validated experimentally |
| Traceability | 100% automated specimen identification | Limited/manual in conventional workflows |

These values should be treated as R&D validation targets rather than established performance claims until they are experimentally demonstrated on representative cable types and sample sizes.

6. Key Research Questions

The development of the system should investigate the following research questions:

- Can sensor-based measurement achieve the required cutting-depth accuracy across different cable insulation and sheath materials such as PVC, XLPE, and HDPE?

- Which sensing and machine-learning approach provides reliable real-time identification of cable insulation/sheath materials?

- Can adaptive cutting-depth control significantly reduce conductor damage compared with fixed-depth or manually adjusted stripping?

- Can sensor fusion between diameter measurement, material identification, encoder feedback, and cutting-force/depth information improve process repeatability?

- Can an inline machine-vision system reliably detect dimensional deviations and visible defects before the specimen enters the testing stage?

- Can automated standards-based parameter selection reduce operator-dependent variation in specimen preparation?

- Can the complete automated workflow reduce specimen preparation time while maintaining or improving dimensional accuracy and repeatability?

7. Key Reference Technologies and Literature

1. Kataleri et al. (2026) – Automated Cable Specimen Preparation Prototype

This work provides a useful reference for automated multi-stage cable specimen preparation and reported a preparation time of approximately 142 seconds with a Cpk value of approximately 1.71.

https://ijirt.org/publishedpaper/IJIRT196451_PAPER.pdf

2. IS 10810 Part 7 – Methods of Test for Cables

This standard provides relevant requirements associated with specimen preparation and dimensions for cable testing.

https://ia601406.us.archive.org/35/items/gov.in.is.10810.7.1984/is.10810.7.1984.txt

3. CN122495247A (2026) – Adaptive Cable Stripping Technology

This patent provides relevant technological background for automated/adaptive cable stripping mechanisms and can be considered when developing the adaptive stripping subsystem.

https://eureka.patsnap.com/patent/CN122495247A

4. CORPEX Machine Vision System

The CORPEX Machine Vision System provides an industrial reference for machine-vision-based inspection of cable-related products.

https://www.corpex.biz/cmvs-corpex-machine-vision-system/

5. IEC 60811-508

IEC 60811-508 provides an international reference for test methods involving cable insulation/sheathing materials and is relevant when considering standardized specimen preparation and testing workflows.

https://cdn.standards.iteh.ai/samples/17387/ddcc34930a3641b284cd20900cb8edc5/IEC-60811-508-2012.pdf

Overall R&D Opportunity

The individual technologies required for automated cable specimen preparation already exist in different forms, including automated wire processing, laser measurement, machine vision, adaptive cutting, servo control, material sensing, and industrial data connectivity.

However, the major R&D opportunity lies in integrating these technologies into a single closed-loop, adaptive, standards-aware cable specimen preparation system.

The proposed system would move the process from conventional manual and operator-dependent specimen preparation toward an intelligent automated workflow consisting of:

Measure → Identify → Adapt → Cut → Strip → Flatten → Slice → Form → Verify → Trace

The core research contribution is therefore not simply the automation of individual operations, but the development of a sensor-fusion and closed-loop control architecture capable of adapting the preparation process to cable geometry and material while automatically verifying the resulting specimen against predefined standards-based requirements.